Suriya Pandi P

251901168

COMPUTER NETWORKS

| Ex.No.10 | Design a simple topology and configure with one router, two switches and PCs using Cisco Packet Tracer |
| --- | --- |
| DATE: |  |

AIM:

To design and configure a simple network topology with one router, two switches, and four PCs in Cisco Packet Tracer for successful inter-network communication.

PROCEDURE:

1. Topology

Router R1

┌────────────┐

│            │

G0/0          G0/1

│            │

│            │

Switch S1    Switch S2

┌────┴────┐  ┌────┴────┐

│         │  │         │

PC1       PC2 PC3       PC4

2. Devices to place

In Cisco Packet Tracer, place:

- 1 Router – Cisco 1941

- 2 Switches – Cisco 2960

- 4 PCs

Use Copper Straight-Through cables.

3. IP Addressing

| Device | Interface | IP Address | Subnet Mask | Gateway |
| --- | --- | --- | --- | --- |
| R1 | G0/0 | 192.168.1.1 | 255.255.255.0 | — |
| R1 | G0/1 | 192.168.2.1 | 255.255.255.0 | — |
| PC1 | NIC | 192.168.1.10 | 255.255.255.0 | 192.168.1.1 |
| PC2 | NIC | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 |
| PC3 | NIC | 192.168.2.10 | 255.255.255.0 | 192.168.2.1 |
| PC4 | NIC | 192.168.2.11 | 255.255.255.0 | 192.168.2.1 |

So we have two different networks:

Network 1: 192.168.1.0/24

PC1 ─┐

PC2 ─┤

S1 ─── R1 G0/0 = 192.168.1.1

Network 2: 192.168.2.0/24

PC3 ─┐

PC4 ─┤

S2 ─── R1 G0/1 = 192.168.2.1

4. Configure Router R1

Click R1 → CLI and enter:

enable

configure terminal

interface gigabitEthernet 0/0

ip address 192.168.1.1 255.255.255.0

no shutdown

exit

interface gigabitEthernet 0/1

ip address 192.168.2.1 255.255.255.0

no shutdown

exit

end

write memory

The important command is:

no shutdown

It activates the router interface.

Check the interfaces using:

show ip interface brief

You should see the interfaces as up/up.

SCREENSHOTS:( to be pasted)

5. Configure PC1

Go to:

PC1 → Desktop → IP Configuration

Enter:

IP Address:       192.168.1.10

Subnet Mask:      255.255.255.0

Default Gateway:  192.168.1.1

SCREENSHOTS:( to be pasted)

6. Configure PC2

IP Address:       192.168.1.11

Subnet Mask:      255.255.255.0

Default Gateway:  192.168.1.1

SCREENSHOTS:( to be pasted)

7. Configure PC3

IP Address:       192.168.2.10

Subnet Mask:      255.255.255.0

Default Gateway:  192.168.2.1

SCREENSHOTS:( to be pasted)

8. Configure PC4

IP Address:       192.168.2.11

Subnet Mask:      255.255.255.0

Default Gateway:  192.168.2.1

SCREENSHOTS:( to be pasted)

9. Test the network

From PC1 → Desktop → Command Prompt, type:

ping 192.168.1.11

This tests PC1 → PC2.

Then:

ping 192.168.2.10

This tests PC1 → Router → Switch 2 → PC3.

You should receive:

Reply from 192.168.2.10

Reply from 192.168.2.10

Reply from 192.168.2.10

Reply from 192.168.2.10

SCREENSHOTS:( to be pasted)

SAMPLE CONFIGURATION:

OUTPUT:

RESULT:

A simple network topology with one router, two switches, and PCs was successfully designed and configured using Cisco Packet Tracer.Connectivity between the network devices and PCs was successfully established and verified.